- Go to the Trinity Downloads page here http://www.diablosport.com/downloads-updates?pid=2

- In the Tool Serial Number field, enter the word JOBBERS and hit Enter.  It will open up a short menu of links to downloadable files.

- Download the 1r11 file (right-click the link and select “save as”) to your Desktop or any folder you can retrieve it from.

- When it is finished downloading, disconnect your computer from its connection to the internet before you proceed.

- With the Trinity unplugged, press and hold the left directional button and hold it while you plug the OBD-II cable and let the tool boot up while holding the button down.

- If it works, the tool will finish booting and display “The Trinity will now walk you through the vehicle selection process”.  Leave it on that screen and open the DS Downloader program.

- Once the program finds the Trinity (Trinity’s screen will display “Tool is under remote control”), click Update Tool and browse for and select the 1r11 .crom file you downloaded earlier.  Click Open and it should start sending the file.

- Once it is complete, you can reestablish your internet connection and rerun the DS Downloader program a second time (just close it and reopen it) to complete the reset of the updates automatically.

NOTE: If the DS Downloader program asks you to register the tool, make sure to do so and it will update automatically from there.