Diablosport [image: image1.png]O




U7146/U7142/U7153 Custom tune installation instruction

Saving original backup to PC
1. Allow the Predator to save a copy of the original backup file from the PCM in the car.

2. Once the backup has been saved, connect the Predator to the PC using a straight through 9-pin serial cable, and power it up using either a 12v 1 amp power supply or the OBDII port in the vehicle if using a laptop.

3. Follow this link to download and install the DSDownloader to your computer. http://diablosport.com/CMR/software/General_Software/DS_Downloader/DSDownloader_Installer_1740.exe
4. Open the DSDownloader software.

5. Access your device manager to see what COM port the cable is assigned to and set the baud rate for that COM port to 57600.

6. Set the DSDownlaoder to the same COM port and baud rate settings. Be sure the compressed send button is not checked on the downloader tool bar..

7. From the main menu in the Predator go to options>custom tune

8. Select export tune and then original backup. You will see a window that says ‘processing’, and it will count up to 100%.

9. Now, press enter on the Predator. It will display ‘sending 0 bytes’.

10. Once you do this, press the ‘get tune file from Predator’ button on the DSDownloader.

11. A new window will open on the PC called Diablosport tool download and it will display a status bar. It should say ‘reading packet’.

12. Once the file has beed uploaded the downloader will ask if you would like to save in the .ori folder. Choose no, and direct it to save the file to your desktop or somewhere that it will be easy for you to find.

13. If you have any issues while following the steps outlined above, be sure you have all of the COM port and baud rate settings correct. If all else fails, please contact our technical support at 561-908-0040.

Sending custom tune to Predator

1. Open the DSDownloader and be sure your COM port and baud rate settings are correct. Be sure the compressed send button is not checked on the DSDownloader tool bar.

2. Connect the Predator to the PC using the method outlined above.

3. From the main menu in the Predator, go to Options>Custom tune.

4. Leave the ‘import custom tune’  option highlighted.

5. On the DSDownloader, press the ‘send tune file to Predator’ button.

6. You will have a new window open up, set this to look at the desktop or where ever you have the custom tune saved.

7. Highlight the custom tune and press open.

8. Now press enter on the Predator twice, and the file will automatically transfer. The Predator will display ‘transfer OK’ once the tune is loaded.

9. Now when you go to the main menu it will read :
Custom tune
Diagnostics
Options

10. Follow the normal procedure for loading the custom tune to the vehicle.

11. If you need assistance with any of the steps outlined above, please contact our technical support at 561-908-0040.


