DCX Vehicle Update Instructions- Predator Already Updated

Follow these instructions if you get the message on your Predator that reads: “This vehicle needs an update Download here: www.diablosport.com/DCX” and your tuner is already on the most current revision. This is because the tool wants to update the vehicle’s stock calibration file, not necessarily because the tool itself needs an update.  We have configured the Predator to be able to run the same update function as a dealership’s equipment, so that you do not need to bring it in to them when the update is required.  However, the tool’s firmware (the software that controls it) may still need an update first.

To determine if the tool’s software is current, boot it up by plugging it in to power, and look at the code next to the word ‘Predator’ on the main screen, it will read “v.#r##”. Compare this to the most recent revision on the Downloads page for your Predator to see if the unit is current.  If it is not, follow the instructions titled, “DCX Vehicle Update Instructions plus Revision Update”.

PREP:

NOTE: DSDownloader does not work with Belkin brand USB-serial cables

Purchase

From:

http://www.diablosport.com, click Products, then Online Store
-buy part #U7778 for PCs that have USB ports, OR U7777 for PC’s that have a serial port
OR
 from Radio Shack, Best Buy etc:
-USB to Serial cable OR a straight 9-pin Serial/Serial cable
 and a
-12 v 1 amp power cable, center pin is positive

Download the Software:

Go to http://www.diablosport.com, click Support, then Downloads, enter Vehicle Type or Predator Part number (U71##):
 Tip: Make sure to download all files by right-clicking the link and selecting “save as”, then put the file on the Desktop before hitting “save”

Driver Programs

The Driver program allows the tool to communicate to the computer through the data cable. USB/Serial cables contain a microchip and the driver simply controls it.

If you are using a USB/Serial cable: Make sure to download the appropriate driver for your version of Windows under "Diablosport USB Serial Drivers". For Windows 7 download the one titled "PL2303_Prolific_DriverInstaller_v110.exe", or under “Old Prolific Drivers” choose the one specific to your version of Windows.

In most cases, the driver will not work until the computer is rebooted. Once you have installed the DSDownloader, installed the driver program and RESTARTED your computer, you may proceed to the first step in the instructions.

If you are using a straight serial/serial cable, you don’t have to install a driver program and reboot the computer. Just install the DSDownloader as described below.

The DSDownloader program

This manages the transfer of information from a computer to a Diablosport tool.   Download and run the installer program in the in the red link titled, “DS Downloader Installer 2.2.1.7.exe".

Tips When running the DSDownloader Installer Wizard

1. Microsoft will tell you “The Publisher could not be verified”. This is normal, just click ‘Yes’.

2. Before the install begins, it will also give you the option to Create Desktop Icon. Make sure to check this box if you want to have the DSDownloader in a convenient place.

3. The DSDownloader will also want to install the Trinity driver program. If you do not have a Trinity, you can let it install if you want, or cancel it. It’s not used for the Predator.

4. You can also delete the installer programs from your Desktop once the actual programs themselves are installed to avoid confusion.

Uploading the Calibration Update

- Connect the tool to the computer with the power and USB/Serial cables.

- Open the DSDownloader program and under ‘Options’ make sure there is a check mark next to the line that says “Auto Connect”.

- The DSDownloader will scan the ports on your computer, and when it detects the tool it will pop a window that says, “Diablosport tool was detected on COM #”. Click ‘Close’ on this window to proceed.

NOTE: If the DS Downloader will not connect to the tool, reference the Driver Troubleshooting below.

- If the software detects that you need a proper update, you will see a new tab at the bottom of the application labeled "Calibration Update".
Click on the bottom labeled "Calibration Update" and look for a button that says "Calibration Updater". Click that button.

- Once this is done, unplug your Predator and bring it to the vehicle.  Plug the cable into your ODBII port and select the Performance menu. The tool will first ask for permission to proceed with the Calibration Update, press the center button to continue. 
After the Calibration Update is complete, you can continue to flash the tunes that are built into the Predator or Trinity.

Driver Troubleshooting:

If the DS Downloader program searches but does not find the tool:

For XP:

Right click 'My Computer'/PROPERTIES/HARDWARE/DEVICE MANAGER/double-click PORTS/ then find the line describing the cable (Prolific USB, Radio Shack whatever...) and double-click it, under DEVICE STATUS it has to read 'This device is working properly' to insure that the driver program is installed.

For Vista:

The procedure is the same as XP, but you can click right to 'DEVICE MANAGER' after right-clicking on the 'Computer' icon.

Note about Vista: if the Device Status reads 'This device cannot start Code 10', that is an issue with Windows and you will have to have someone diagnose the problem (like the Geek Squad or equivalent), or try another computer. However, sometimes uninstalling and reinstalling the driver program will solve this issue.

Click the Start Menu and select ‘Control Panel’, then Device Manager, and find the line describing the cable (Prolific USB, Radio Shack whatever...), double-click it, and under DEVICE STATUS it has to read 'This device is working properly' to insure that the driver program is installed.

OR

If your version of Windows 7 does not have a ‘Control Panel’ selection in the START menu, you can select ‘Devices and Printers’, and the port the cable is plugged into will be listed under the ‘Unspecified’ heading. Double-click that icon, then the ‘Hardware’ tab. The cable will be listed under ‘Device Functions’, double-click that line to see the Device Status. It should read 'This device is working properly' to insure that the driver program is installed.

If the driver program is installed (Device Status reads “Device is working properly”) and the tuner is not connecting:

It is likely a bad cable or faulty USB port.

If you have any along the way, please contact our technical support at 800-531-5484 (option 1), visit our technical support forums at www.diablosport.com, or email support@diablosport.com.

-Tech Support