Revision Update Instructions- Using Legacy Connect

Some older versions of Predator software cannot be auto detected by the DS Downloader. Any tool with a revision starting with 7rxx and some older 8rxx revisions will fall into this category. Any tool with a 9rxx revision will auto connect, and should not require legacy connection (but it can still be utilized if you desire). If your tool requires Legacy connection, follow these steps.  Also for use if your tool displays “Application is Missing or Corrupt”

PREP:

NOTE: DSDownloader does not work with Belkin brand USB-serial cables

Purchase

From:

http://www.diablosport.com, click Products, then Online Store
-buy part #U7778 for PCs that have USB ports, OR U7777 for PC’s that have a serial port
OR
 from Radio Shack, Best Buy etc:
-USB to Serial cable OR a straight 9-pin Serial cable
 and a
-12 v 1 amp power cable

if you do not already have the interface and power cables.

Go here for the files:
http://www.diablosport.com, click Support, then Downloads, enter Vehicle Type or Predator Part number (U71##):
 (Make sure to download all files by right-clicking the link and selecting “save as”, then put the file on the Desktop before hitting “save”)

If you are using a USB/Serial cable, make sure to download the appropriate driver for your version of Windows under "Diablosport USB Serial Drivers". For Windows 7 download the one titled "PL2303_Prolific_DriverInstaller_v110.exe", or under “Old Prolific Drivers” choose the one specific to your version of Windows

Note: If you are using a straight serial/serial cable, you don’t have to install the driver program.

and the DSDownloader installer program [the red link titled, “DS Downloader Installer 2.2.1.7.exe"].

Once you have installed the DSDownloader, installed the driver program and RESTARTED your computer, you may proceed to the first step in the instructions.

Installing Update File

1. On the tool select OPTIONS/SHOW LAST TUNE WRITTEN. It must read ‘Original Backup’. If it does not, you must return the vehicle back to stock.  This is accomplished by connecting the tool to the vehicle, selecting Performance, then ‘Original Backup’ as the tune to install, thus returning the PCM back to stock. If you are unable for whatever reason to restore the stock tune to the vehicle, please contact technical support at the information below before proceeding.
2. Power up your Predator and connect it to the PC.

3. Launch the DSDownloader from the desktop icon (looks like our logo on a white background).

4. Once DSDL opens up, you’ll see a screen that prompts you to select the Tool Communication Configurations. Select tool type (Predator) and select the current revision level then click ‘Accept Modifications’.

5. On the DS Downloader click ‘Options’ then select ‘Legacy Connect’. At this point, you will see 2 drop down windows at the top of DSDownloader.

6. To determine which COM port to set the downloader to look at, we need to know what COM port the cable has been assigned to (if using a straight through serial to serial cable, it will generally be COM1).

7. To find what COM port it is, you must check it in the computer’s Device Manager:

For Windows XP:
Right click 'My Computer'/PROPERTIES/HARDWARE/DEVICE MANAGER/double-click PORTS/ then find the line describing the cable (Prolific USB, Radio Shack whatever...) and double-click it, under DEVICE STATUS it has to read 'This device is working properly' to insure that the driver program is installed. 
For Windows Vista:

The procedure is the same as XP, but you can click right to 'DEVICE MANAGER' after right-clicking on the 'Computer' icon. 
      Note about Vista: if the Device Status reads 'This device cannot start Code 10', that is an issue with Windows and you will have to have someone diagnose the problem (like the Geek Squad or equivalent), or try another computer. However, sometimes uninstalling and reinstalling the driver program will solve this issue.
For Windows 7:

Click the Start Menu and select ‘Control Panel’, then Device Manager, and find the line describing the cable (Prolific USB, Radio Shack whatever...), double-click it, and under DEVICE STATUS it has to read 'This device is working properly' to insure that the driver program is installed.
OR

If your version of Windows 7 does not have a ‘Control Panel’ selection in the START menu, you can select ‘Devices and Printers’, and the port the cable is plugged into will be listed under the ‘Unspecified’ heading. Double-click that icon, then the ‘Hardware’ tab. The cable will be listed under ‘Device Functions’.  Double-click that line to see the Device Status. It should read 'This device is working properly' to insure that the driver program is installed.

8. In the Device Manager, you must also set the Bits per Second rating to 115200 (click OK to save the change), then take note of the port (COM#) that the cable is plugged into. You can then exit the path to the Device Manager.

9. On the DSDownloader window, set the drop down window on the DSDL to the same COM port number, and make sure the baud rate is set to 115200.

10. Now, click the ‘Express Tune’ Tab at the bottom of the DSDownloader.

11. From here, click the ‘Update tool’ button.

12. Now you’ll see a ‘firmware file to download’ window. Browse the folder where you saved the .crom file and highlight it, then click Open.

13. A new window will appear titled ‘DiabloSport firmware download’, asking you to power cycle the tool to continue. Unplug the unit from the power source and plug it right back in.

Troubleshooting Note: If the driver program is installed (Device Status reads “Device is working properly”) and the DSDownloader is not sensing the power cycle

It is likely a bad cable or faulty USB port.

14. Once the unit powers back up, the DSDL will see the power up message and tell you that ‘the tool is ready, do you wish to download firmware?’, click yes, and the DSDownloader will send the crom file to the tool.

15. The DSDownloader will show you the file transfer status. Once the transfer is complete, a message will appear stating ‘download complete’ At this point, do not unplug the tool yet.

16. Watch the tool, it will reboot once the file has finished transferring. If it is a Predator, wait to see the new revision displayed at the top line of the main menu before unplugging it. At this point, you may take the tool to the vehicle and install the Performance tune.

NOTE TO CHRYSLER/DODGE OWNERS:
If you get a message that reads "this vehicle needs an update, Download here : www.diablosport.com/dcx", DON'T PANIC, this is because the tool wants to update the vehicle’s stock calibrations file. Go back under the Options menu on the DS Downloader, check ‘Auto Connect’, and follow the instructions titled, “DCX Vehicle Update Instructions-Tuner Already Updated”.

If you have any along the way, please contact our technical support at 800-531-5484 (option 1), visit our technical support forums at www.diablosport.com, or email support@diablosport.com.

-Tech Support