[image: image1.jpg]



05-up Ford Predator custom tune instructions

These instructions will guide you through the process of installing a custom tune on your 05-newer Ford Predator.

These instructions will cover the following part numbers:

U7140, U7145, U7151, U7155, VR7140, VR7145, VR7151, PC7140, PC7145, PC7151, SLP7140, SLP7145, ST7149, CL7140

Required Items

1: Serial Cable:

Standard straight-through DB9 (9-pin to 9-pin) RS232C serial cable  (6 foot cable is more than enough). Check your computer to see if you have a male or a female port on the back, as some are different. You will need a male end to go into your Predator.
(Radio Shack Part # 26-117, 6-ft DB9 male to DB9 female shielded cable.)


2: Power Adapter:

CLASS 2 TRANSFORMER
OUTPUT : 12VDC 1000mA (1 Amp)(Radio Shack Part # 273-1776)
Note: Plug Tip is 2.5mm x 5.5mm, female, and center is positive.(Radio Shack Part # 273-1717)

Radio Shack normally carries both of these items. Most Wal-Marts have the Power Adapter (multi output version).

Note: If you have a laptop, you can use the vehicle's power to power up the Predator, instead of using a Power Adapter, and just do the following with your laptop and Predator inside of your vehicle.

Saving original backup to PC:

1. Go to our website at www.diablosport.com and click downloads from the left hand side of the screen.

2. Select part number U7140 or U7145. You will automatically be directed to the download page for that part number.

3. Go to the bottom of the page and download the DSDownloader installer. (Right click, save target as).

4. Run the program and go through the install wizard to set up the DSDownloader. Allow it to set up a desktop icon for easy access to the software.

5. Once the install is complete, run the DSDownloader program. You will get a message box saying ‘not registered’. This is normal, just click ‘OK’.

6. You will now need to check to see what COM port your serial or USB cable is assigned to in the device manager. On the bottom left hand side of your computer screen click the ‘START’ button.

7. From this menu, right click on ‘my computer’, then go to properties.

8. Select the hardware tab, then click the ‘device manager’ button.

9. In the device manager, go to Ports (COM and LPT) and click the plus sign (+) next to ports.

10. If you are using a 9-pin to 9-pin serial cable, double click on ‘communication port COM1’. If using a USB to serial adapter, double click on the device when it drops down under Ports.

11. Click the port settings tab.

12. Set the baud rate (bits per second) to 115200.

13. Close out the Communication port properties, the device manager and the System properties windows. 

14. Set the options on the DSDownloader to match the COM port that your cable is assigned to in the device manager.

15. Set the baud rate on the DSDownloader to 115200.

16. Power up the Predator with the 12v power supply and connect the serial port to the bottom of the tool.

17. Once the tool has booted up and is on the main screen, press ‘get tune file from Predator’ on the DSDownloader.

18. A copy of the original backup file will automatically be extracted from the Predator.

19. Once the file has been uploaded, the DSDownloader will ask if you would like to save it in the .ori file directory. Choose ‘no’ and direct it to save the file to your desktop or somewhere that you can find it easily.

20. Your original backup file is now saved to your PC.

Sending a custom tune to the Predator:

1. Open up the DSDownloader and connect the Predator to the PC and power it up.

2. Be sure all of your COM port and baud rate settings are correct as per steps 6-15 from above. Also be sure the ‘compressed send’ button is checked on the top of the DSDownloader.

3. On the DSDownloader press the ‘send tune file to Predator’ button.

4. You will get a new window asking you to select the file for download.

5. Browse through the computer to find where you saved the custom tune, and click it once, then press ‘open’.

6. If this is the first time you are sending a tune to the Predator, you will get a new window saying a valid base file could not be found. Click ‘Yes’.

7. Now, direct the DSDownloader to look in your desktop or wherever you saved the backup file and highlight it, then press open.

8. The DSDownloader will automatically compare these files to determine if they are the same size and of the same calibration.

9. Once it determines the files are OK, it will download the file to the Predator. This will all happen very fast, usually in under 10 seconds.

10. The DSDownloader will tell you that the download was successful, and the Predator will say ‘import OK’.

11. You may now unhook the Predator and go to the car to install the custom tune.

If for any reason you cannot upload the backup file or download a custom tune to the Predator after following these instructions, please call our tech support department at 561-908-0040.

