[image: image1.jpg]



Predator/Trinity revision update instructions
Note: These instructions are based on the assumption that your Predator or Trinity is somewhat up to date. If the revision history of the tool (Predator only) is unknown, or if the Predator has a revision that displays 7rXX then please proceed to the 2nd set of instructions for using legacy connect.
To update a Predator or Trinity unit, you will need a USB to serial cable (Predator) or the provided USB cable (Trinity), and you’ll need to install the appropriate driver (USB to serial cable driver or Trinity driver that came with the CD in your Trinity package). You will also need to download from our website (www.diablosport.com) the latest revision update file (.crom file) and the DSDownloader installer. Once you have installed the DSDownloader and saved the update file to your PC, you may proceed to the first step in the instructions.
1. Before updating your Predator or Trinity, you MUST restore the stock tune to the vehicle. This is accomplished by selecting the ‘Original Backup’, and installing it in the vehicles PCM, thus returning the PCM back to stock. If you are unable for whatever reason to restore the stock tune to the vehicle, please contact technical support at 561-908-0040 prior to attempting to update the tool.
2. Power up your Predator or Trinity and connect it to the PC.
3. Launch the DSDownloader from the desktop icon (looks like our logo on a white background).
4. Once DSDL opens up, you’ll see a screen that prompts you to select the tool communication configurations. Select tool type (Trinity or Predator) and select the current revision level if it is a Predator, then click accept modifications.
5. At this point, DSDL will start searching for your tool on the COM ports. Once it finds the tool, you’ll get a pop up that states the tool was found on COM _ at a baud rate of 115200. Click close here to proceed.
6. DSDL will now take you to the express tune tab. From here, click the ‘Update tool’ button.
7. Now you’ll see a ‘firmware file to download’ window. Browse to where you saved the .crom file and select it, then click Open.
8. A new window will appear (Predator only, Trinity users, skip to next step) titled ‘DiabloSport firmware download’, asking you to power cycle the tool to continue. Unplug the unit from the power source and plug it right back in.
9. Once the unit powers back up, the DSDL will see the power up message and tell you that ‘the tool is ready, do you wish to download firmware?’, click yes, and the DSDownloader will send the crom file to the tool.
10. The DSDownloader will show you the file transfer status. Once the transfer is complete, a message will appear stating ‘Please wait for tool to reboot’ At this point, do not unplug the tool yet.
11. Watch the tool, it will reboot once the file has finished transferring. If it is a Predator, wait to see the new revision displayed at the top line of the main menu before unplugging it, if Trinity, wait until it comes back to the main menu. At this point, you may take the tool to the vehicle and install the Performance tune.
Using Legacy connect
Some older versions of Predator software cannot be auto detected by the DSDownloader. Any tool with a revision starting with 7rxx and some older 8rxx revisions will fall into this category. Any tool with a 9rxx revision will auto connect, and should not require legacy connection. If your tool requires Legacy connection, follow these steps.
1. Before updating your Predator or Trinity, you MUST restore the stock tune to the vehicle. This is accomplished by selecting the ‘Original Backup’, and installing it in the vehicles PCM, thus returning the PCM back to stock. If you are unable for whatever reason to restore the stock tune to the vehicle, please contact technical support at 561-908-0040 prior to attempting to update the tool.
2. Power up your Predator and connect it to the PC.
3. Launch the DSDownloader from the desktop icon (looks like our logo on a white background
4. Once DSDL opens up, you’ll see a screen that prompts you to select the tool communication configurations. Select tool type (Predator) and select the current revision level then click accept modifications.
5. At this point, you will see 2 drop down windows at the top of DSDownloader.
6. To determine which COM port to set the downloader to look at, we need to know what COM port the cable has been assigned to. (if uning a straight through serial to serial cable, it will generally be COM1)
7. To find what COM port it is, click the start button on the PC, and right click on My Computer (Computer on Vista), and select properties. Click the hardware tab, then the device manager button (Vista just click the device manager button).
8. In the device manager, go to Ports, and click the + sign next to ports. You should see your USB cable listed here, with the assigned COM port listed in parenthesis (COM 5 for ex.). (If you do not see ports, it may be under ‘other devices’).
9. Take note of the COM port it is assigned to, then set the drop down window on the DSDL to that COM port number, and make sure the baud rate is set to 115200.
10. Now, click the ‘Express Tune’ Tab at the bottom of the DSDownloader.
11. From here, click the ‘Update tool’ button.
12. Now you’ll see a ‘firmware file to download’ window. Browse to where you saved the .crom file and select it, then click Open.
13. A new window will appear titled ‘DiabloSport firmware download’, asking you to power cycle the tool to continue. Unplug the unit from the power source and plug it right back in.
14. Once the unit powers back up, the DSDL will see the power up message and tell you that ‘the tool is ready, do you wish to download firmware?’, click yes, and the DSDownloader will send the crom file to the tool.
15. The DSDownloader will show you the file transfer status. Once the transfer is complete, a message will appear stating ‘download complete’ At this point, do not unplug the tool yet.
16. Watch the tool, it will reboot once the file has finished transferring. If it is a Predator, wait to see the new revision displayed at the top line of the main menu before unplugging it. At this point, you may take the tool to the vehicle and install the Performance tune.
If you have any issues with updating your DiabloSport tool, please contact our technical support at 800-531-5484x1 or visit our technical support forums at www.diablosport.com
